QUESTION BANK: UNIT - II ELECTRICITY & MAGNETISM

Course: First Year Diploma Engineering

# Section A: 4-Mark Questions (Short Answer & Direct Numerical)

## Part I: Theory Questions

Define Electric Charge. State its SI unit and list any two basic properties of charges.

State Coulomb’s Inverse Square Law of electrostatics and write its mathematical equation with meaning of each term.

Define Electric Field and Electric Field Intensity (E). State the SI unit of Electric Field Intensity.

Define Magnetic Field and Magnetic Flux (Φ). Write the SI units for both.

State any four standard properties of Magnetic Lines of Force.

State Ohm’s Law. Draw a neat circuit diagram used to verify Ohm's law experimentally.

Define Specific Resistance (Resistivity). State its formula and its SI unit.

## Part II: Numerical Questions

Two point charges of +4 μC and +6 μC are placed in air at a distance of 0.2 meters from each other. Calculate the electrostatic force acting between them. (Given: 1/(4πε₀) = 9 × 10⁹ N·m²/C²).

Calculate the electric potential at a point located 0.15 meters away from a single point charge of +3 μC in air.

A uniform magnetic flux of 1.2 × 10⁻³ Wb passes normally through a rectangular coil of dimensions 10 cm × 20 cm. Find the magnetic flux density of the field.

A wire of resistance 20 Ω is connected across a 12V battery. Calculate the current flowing through the wire and the total electric charge passing through it in 2 minutes.

# Section B: 6-Mark Questions

## Part I: Theory Questions

Difference bet ammeter and voltmeter .

## Part II: Numerical Questions

Three resistors of values 5 Ω, 10 Ω, and 15 Ω are connected in parallel. This entire combination is then connected in series with a 4 Ω resistor across a 24V DC supply. Calculate the total equivalent resistance of the circuit and the main supply current.

A galvanometer has a wire resistance of 50 Ω and shows a full-scale deflection for a current of 2 mA. Calculate the value of resistance needed to convert this galvanometer into:
(a) An ammeter reading up to 5 A.
(b) A voltmeter reading up to 10 V.

# Section C: 8-Mark Questions

Question 1

Part A (4 Marks): State Coulomb's Law of electrostatics. Explain how the force between two fixed charges changes if the air medium between them is completely filled with a dielectric medium of relative permittivity (dielectric constant) K = 4.

Part B (4 Marks): Two identical isolated metallic spheres carrying charges +2 μC and -6 μC are placed at a certain distance apart in air. They are brought into brief physical contact with each other and then returned to their exact original positions. Analyze and calculate the ratio of the electrostatic force.

Question 2

Part A (4 Marks): Define Specific Resistance. Explain how the specific resistance of a material changes with (a) an increase in its temperature, and (b) changing its cross-sectional dimensions.

Part B (4 Marks): An engineering lab experiment requires a heating element that can generate 36,000 Joules of heat energy per minute when connected to a 200V power source. Calculate the required operational resistance of this heating wire.

Question 3

Question 4

Part A (4 Marks): State the laws governing series and parallel combinations of resistances. Identify which type of combination is utilized for standard domestic household wiring and provide a sound engineering reason why.

Part B (4 Marks): For the network layout description: A resistor of 3 Ω is connected in series with a parallel combination of two resistors having values 6 Ω and 12 Ω. If a total potential difference of 36V is maintained across the entire system layout, calculate the current passing through the parallel branch of 6 Ω.